| Parameter | Value | Remarks |
| --- | --- | --- |
| Product Code | ACS37030/32 |  |
| Product Type | Current Sensor |  |
| Package dimensions | 4.9 x 6 mm | Useful in smaller robots like those used for cleaning, quadrotors etc. Compact 6-pin SOIC package |
| Bandwidth | DC to 5MHz |  |
| Response time | 40ns |  |
| Sensing Range/s | +/- 20A, +/- 40A, +/- 60A |  |
| Sensitivity over temperature (as %) | +/- 2% |  |
| Noise | 50mA - RMS | Low input noise |
| Grade | AEC-Q100 Grade 0 | Automotive standard |
| Power consumption | 20mA at 3.3V |  |
| Internal resistance | 0.68mΩ | Low internal conductor resistance is an advantage as it leads to lower IR losses since battery life is often a major design constraint. |
| Notes: - Offers a dedicated reference voltage output (VREF) for differential routing which gives a crucial advantage in electrically noisy environments. - Dedicated fault output for overcurrent detection. |  |  |